# Tube furnace to grow carbon nanotubes

Company: Lindberg/Blue

Product name: Mini-Mite

Type: UP150 controller

How to operate this furnace

Argon flow only:

Make sure that the quartz tube is inside the oven and that the oven is closed. Attach the gas insert flow pipe to the right end of the quartz tube and the gas outlet flow pipe to the left end of the quartz tube. Open the Ar cylinder and adjust the flow.

## How to enter a program into the controller

See chapter 7, page 7 and 8 of the manual (in the metal cupboard in 702)!

First press the SET/ENT key for 3 seconds until you see PrG in the upper display and 0 in the lower.

Press the  run button until you see –1 in the lower display, press  enter.

Now you have entered the programming mode.

This is an example of a heating and cooling program:

SSP

SP1

SP3

SP2

SP4

SP5

Tm1

SEG 1

Tm2

SEG 2

Tm3

SEG 3

Tm4

SEG 4

Tm5

SEG 5

This program heats the oven up from the starting setpoint (SSP) to SP1 within the time interval Tm1. The temperature then stays constant during Tm2 because SP2 = SP1. Heating starts again during Tm3 until the temperature of SP3 has been reached. Now we are again at a segment of the program where the temperature is kept constant (SP3 = SP4), during Tm4. In segment number 5 the oven is cooled to SP5 within Tm5.

The red parameters in the scheme are the parameters that you have to attribute a value to in the program.

Now follow the instructions of part 7.3 of the manual. Do not edit any other value than PrG, SSP, SP1, 2,3,… and Tm1,2,3,…

To end the program, enter off for the next TM (in the example: Tm6 = off).

TO START THE PROGRAM, PRESS THE  run BUTTON until you hear a click!!!

To check which segment of the program is active, press the SET/ENT key. You see that the SEG signal lightens up and by pressing the SET/ENT key again, you can scroll from information about segment number, how much time this segment still takes and to which temperature it will bring the oven.

After you have finished, please switch off the oven. Cooling down goes much faster when you open the oven, but please close it again when it is cold, to avoid damage to the quartz tube.