Carbon nanotube AFM tip functionalisation

# Karen Bol: 4355

Molecular weigth of mannose+spacer is 223.2 g/mol

(Molecular weight of mannose is 180.2)

Place the tip in solution of 50 mM EDC 1-ethyl-3-(3-dimethylaminopropyl) carbodiimide hydrochloride [C8H17N3.HCl] (in freezer in 702)

and 5 mM mannosamine (Karen)

in 0.1 M MES buffer pH 6 (11th floor)

for 2 hours.

Preparation:

First make a MES buffer with pH = 6

Fw of MES is 195.2 g/mol

0.1 M * 195.2 g/mol = 19.52 g/l				in 100 ml:	1.952g

Fw of EDC is 191.7 g/mol

50 mM EDC * Fw= 50 mM*191.7 g/mol = 9.585 g/l	in 100 ml:	0.9585g

NB: Eerst op kamertemperatuur laten komen, dan pas openmaken!!!

Fw of mannosamine is 215.6 g/mol

5 mM * 215.6 g/mol = 1.078 g/l				in 100 ml:	0.1078g

Also:

0.1 M solution of MES					in 100 ml:	1.952g

0.1 M solution of NaCl					in 100 ml:	5.844g

The tips are then successively washed in 0.1 M solultions of MES, NaCl and deionized water.

The fluid cell for the functionalisation has a volume of 2.5 ml, so around 3 ml is needed per functionalisation event. This means 3.3 mg mannose+spacer.

Karen: 1 ml van een 5 mM oplossing van de suikers is voldoende voor een tip-beladingsexperiment. zal ik van de volledig ontschermde mannose+spacer 0.5 mmol maken? dan heb je in principe genoeg voor 100 ml oplossing.

Figure:

From: Wong et al.

Nature 394, 52-55 (1998)

Karen Bol

Gijs van der Marel